| Repeat Skill Final : CCNA Exploration- 8th Batch, April – 2010 CCNA Exploration 4: Accessing the WAN |  |
| --- | --- |
| Name : ………..……………………… User Id : ……………………………… | Total Marks: 100 Time: 90 minutes |

Configure the network in the accompanying packet tracer file according to the following specification and requirements.

| IP and Link Layer Addresses for the Routers: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Name | Interface |  | IP address |  |
| Central | Fa0/0.10 |  | 192.168.11.1/25 |  |
|  | Fa0/0.20 |  | 192.168.11.129/25 |  |
|  | Ser 0/0/0 |  | 80.80.80.1/28 |  |
| ISP | Fa0/0 |  | 100.100.100.1/30 |  |
|  | Ser 0/0/0 |  | 80.80.80.2/28 |  |
|  | Ser0/0/1.102 |  | 102.102.102.1/30 interface dlci number 102 |  |
|  | Ser0/0/1.103 |  | 103.103.103.1/30 interface dlci number 103 |  |
| Yahoo | Ser0/0/0.201 |  | 102.102.102.2/30 interface dlci number 201 |  |
|  | Fa0/0 |  | 200.200.200.1/30 |  |
| Tele Router | Ser0/0/0.301 |  | 103.103.103.2/30 interface dlci number 301 |  |
|  | Fa0/0 |  | 50.50.50.1/30 |  |
| IP Addresses for the Servers and Hosts: |  |  |  |  |
| EEE2 |  | 192.168.11.32/24 (preconfigured) |  |  |
| Yahoo Server |  | 200.200.200.2/30 (preconfigured) |  |  |
| Google Server |  | 100.100.100.2/30 (preconfigured) |  |  |
| ECE Web |  | 192.168.11.254/25 (Preconfigured) |  |  |
| Tele PC |  | 50.50.50.2/30 (preconfigured) |  |  |

Frame Relay Configuration:

- Configure the four frame relay clouds FRSwitch1, FRSwitch2 and FRSwitch3 to establish the following connections:

  - A point to point link from ISP to Yahoo.

  - A point to point link from ISP to Tele.

Other Requirements:

- Central has bought the real IP series 80.80.80.0/28 from ISP.

- All routers except Central will be configured with OSPF (Process Id: 1, Area Id: 0). Use appropriate wildcard mask for all networks.

- Central router will use a static route through ISP for outside (Internet) access.

- Ser0/0/0 of ISP will be DTE, Ser0/0/0 of Central will be DCE (clock rate = 64000).

- PPP will be enabled between Central and ISP with CHAP authentication. Use cisco as the authentication password.

- Google Server will also act as a DNS server with the following DNS entry-

| Domain Name | IP Address |
| --- | --- |
| www.eceweb.com | 80.80.80.14 |
| www.google.com | 100.100.100.2 |
| www.yahoo.com | 200.200.200.2 |

- Router Central will be configured as a DHCP server for CSE and EEE LANs. The details are as follows:

| DHCP Pool Name | DHCP Network | Excluded Addresses | DNS Server |
| --- | --- | --- | --- |
| CSE | 192.168.11.128/25 | 192.168.11.129-192.168.11.134 192.168.11.254 | 100.100.100.2 |
| EEE | 192.168.11.0/25 | 192.168.11.1-192.168.11.32 | 100.100.100.2 |

- Workstations from CSE and EEE LANs will use the NAT feature to access outside networks. Therefore, configure Central router for NAT translation. The details are as follows:

  - NAT pool name will be POOL and its IP range is 80.80.80.3/28 to 80.80.80.13/28.

  - Use named standard access list NAT for mapping local two LANs to outside usable real IP range.

  - Enable NAT overload feature for dynamic translation.

  - Use static NAT for the server ECE Web (192.168.11.254) so that outside users can browse the server using its global IP address (80.80.80.14).

- Write an Extended Named ACL ‘FILTER’ in Central router and apply on the appropriate interface that satisfies the following requirements:

  - Hosts from EEE LAN in the IP range 192.168.11.33 to 192.168.11.47 will not be able to browse any web server other than the ECE Web. (Try to write the ACL optimally)

  - All other communications must be permitted.

Tasks:

- Browse Yahoo Server (www.yahoo.com) from CSE. (Status: Successful)

- Browse ECE Web (www.eceweb.com) from PC Tele PC. (Status: Successful)

- Ping Google Server (www.google.com) from PC Tele PC. (Status: Successful)

- Ping Yahoo Server (www.yahoo.com) from EEE2. (Status: Successful)

- Browse Google Server (www.google.com) from EEE1. (Status: Failed)

- Browse ECE Web (www.eceweb.com) from EEE1. (Status: Successful)

- Ping CSE from EEE. (Status: Successful)

- Browse Yahoo Server (www.yahoo.com) from CSE. (Status: Successful)